BTEC Level 3 National in IT — Unit 1: Information Technology Systems
Topic: Secondary Storage Devices (Magnetic, Optical, and Solid-State)
STUDENT WORKSHEET

Name: ____________________________		Date: ____________________________

# 📘 Part 1: Core Revision Notes

Secondary storage devices are peripheral hardware components used to store, back up, and share data. Unlike primary memory (RAM), secondary storage is non-volatile, meaning it retains its data permanently even when the computer system is powered off.

In BTEC Unit 1, you must evaluate storage media using key characteristics: Capacity, Speed, Portability, Durability, Reliability, and Cost.

## 1. Magnetic Storage (e.g., Hard Disk Drives - HDD, Magnetic Tape)

How it Works: Uses mechanical platters coated with magnetic material. A read/write head changes the magnetic polarity of sections on the disk to represent binary 1s and 0s.

Characteristics & Limitations:

Capacity: Very large capacity, easily hitting multiple Terabytes (TB).

Cost: Exceptionally low cost per gigabyte/terabyte, making it the cheapest form of mass internal storage.

Speed & Durability: Contains moving parts. This makes HDDs slower than SSDs and highly vulnerable to physical damage or data corruption if dropped or shaken.

## 2. Optical Storage (e.g., CDs, DVDs, Blu-ray Discs)

How it Works: Uses high-powered lasers to burn microscopic "pits" into a reflective surface. A lower-powered laser then reads the disc by bouncing light off the flat surfaces ("lands") and pits; the changes in reflected light are translated into binary.

Characteristics & Limitations:

Portability & Cost: Discs are flat, lightweight, highly portable, and very cheap to manufacture or distribute individually.

Capacity: Low compared to modern media (CDs max out at ~700MB, DVDs at 4.7GB, and Blu-rays around 25GB–50GB).

Durability: Susceptible to scratches and degradation from sunlight or heat, which can easily corrupt data.

## 3. Solid-State Storage (e.g., SSDs, SD Cards, USB Flash Drives)

How it Works: Built using electronic flash memory chips. Data is stored electronically by trapping electrons in floating-gate transistors, meaning there are absolutely no moving parts.

Characteristics & Limitations:

Speed: Exceptionally fast read and write speeds, significantly lowering system boot and application load times.

Durability & Power: Highly durable against knocks or falls because it has no mechanical parts. It also consumes less power, extending laptop battery life.

Cost & Longevity: Considerably more expensive per gigabyte than HDDs. Flash memory also degrades after a finite number of write/erase cycles.

# 📊 Task A: Storage Matrix Fill-in-the-Gaps

Complete the missing characteristics in the table below using the revision notes.

| Storage Category | Common Formats | Speed | Capacity | Durability | Relative Cost |
| --- | --- | --- | --- | --- | --- |
| Magnetic | Internal/External HDD | Moderate | (1) _________________ | Poor if dropped | (2) _________________ |
| Optical | CD, DVD, Blu-ray | (3) _________________ | Very Low (700MB - 50GB) | (4) _________________ | Low per disc |
| Solid-State | SSD, SD Card, USB Drive | (5) _________________ | Moderate (250GB - 4TB) | (6) _________________ | (7) _________________ |

# 📝 Task B: Exam-Style Practice Questions

1. Define the term 'non-volatile memory' in relation to secondary storage. (2 Marks)

2. Explain why an SSD is typically chosen over an HDD for a rugged laptop used by field engineers. (3 Marks)

3. Describe how data is written to and read from an optical disc. (4 Marks)

## Scenario-Based Application

Progressive Media is a small video-production agency upgrading their IT infrastructure. They need storage solutions for three distinct purposes:
• Purpose 1: To act as the main boot drive for their high-performance 4K video editing workstations.
• Purpose 2: To act as a central, high-volume server storage system to archive thousands of hours of finished client footage.
• Purpose 3: To physically hand over final marketing videos to external clients who lack strong internet connections.

4. For each of the three purposes listed above, identify the most appropriate type of storage device (Magnetic, Optical, or Solid-State) and justify your choice by evaluating its performance characteristics. (2 Marks per purpose - 6 Marks Total)

• Purpose 1 Storage Type: _________________________________________
  Justification: ____________________________________________________________________
__________________________________________________________________________________

• Purpose 2 Storage Type: _________________________________________
  Justification: ____________________________________________________________________
__________________________________________________________________________________

• Purpose 3 Storage Type: _________________________________________
  Justification: ____________________________________________________________________
__________________________________________________________________________________